Snap placement measurement - 2” in, halfway down- marked stud and socket

Grommet Placement Measurement – 5 “ from end, 2/3 down

Doted gray lines for furniture

Standard measurements like floor clearance

Color & line standards:

| Element | Color | Style |
| --- | --- | --- |
| Fabric drape | fill #eef2ff, stroke #0f172a | solid 2px |
| Furniture outline (chair/couch under the fabric) | #94a3b8 (gray) | dashed 6,4, 1.5px |
| Corner drape indicators (where drape begins from furniture corners) | #94a3b8 (gray) | dashed 6,4 |
| Seams | #9333ea (purple) | dashed 8,3, 2.5px |
| Cut lines | #dc2626 (red) | solid 2px |
| Grommets | #b45309 (amber) | hollow circle r=6 with r=2 dot inside |
| Snaps | #22c55e fill / #166534 stroke (green) | solid circle r=4 |
| Bungee tabs | #0ea5e9 fill / #0369a1 stroke (sky blue) | diamond with small "T" mark, label "BT" |
| Overall dimensions | #0f172a (black) | bold 13px |
| Sub-dimensions | #64748b (slate) | bold 10px |
| Grommet/snap/tab labels | matching family color | bold 9px |

Layout rules:

- Furniture outline drawn as a gray dashed rectangle inside the fabric polygon, so the manufacturer can see what's furniture vs. what's draping.

- Mirror-aware: grommet dimensions labeled on left side, snap dimensions on right side, each shown once.

- Overall fabric width/height arrows on the outer perimeter; sub-dimensions (drop, drape extents) on the same edge as separate sub-arrows.

- Top of diagram labeled "▼ FRONT" so orientation is unambiguous.